Experiment # 01

LED – Running Lights

Procedure:

- Connect the LED in linear form to the breadboard.  Connect the negative pin of the LED to the GND of the Arduino Uno Board.

- Put 100 Ohms in the positive pin of the LED.  The other end of the resistor will be connected to the 0-13 pins of the Arduino Uno board.

- In the Programming, declare inside the setup function all the pins 0-13 as an output port.

- In the loop function, make at least 5 patterns of your running lights.

Illustration:

Observations:

- There are some issues when there is a wire connected on pin 0 RX and pin 1 TX during the uploading process. This was resolved by taking out the wire on the pin 0 and 1 before the upload then connecting it back after the upload is done.

- Sometimes the Light Emitting Diodes (LEDs) blinks randomly that is not coded on the Arduino. Possible reasons are the following:

- There is a short or inductance that affects the wires.

- The pins of the wires, resistors, or LEDS are sometimes being moved and disconnected from the Arduino or breadboard.

- There is a problem on the internal connections of the breadboard.